UD85C08 8DI-8DO USB Relay Commamd

This product supports two kinds of instructions, AT command (ASCII code) and 8-byte command (HEX format).

Automatic recognition of two kinds of commamds,no need to switch..

The serial port parameters are fixed: 9600,N,8,1

AT command (ASCII code)

Note:AT commands must be uppercase, lowercase invalid.

9600 Band ,8 Data bits,None Parity,1 Stop Bit. This commands has a delay function

Read Status:

Channel 1: AT+R1

Channel 2: AT+R2

Channel 3: AT+R3

Channel 4: AT+R4

Channel 5: AT+R5

Channel 6: AT+R6

Channel 7: AT+R7

Channel 8: AT+R8

Open(ON) :

Channel 1 : AT+O1

Channel 2 : AT+O2

Channel 3: AT+O3

Channel 4: AT+O4

Channel 5: AT+O5

Channel 6: AT+O6

Channel 7: AT+O7

Channel 8: AT+O8

Close(OFF):

Channel 1 : AT+C1

Channel 2 : AT+C2

Channel 3 : AT+C3

Channel 4 : AT+C4

Channel 5: AT+C5

Channel 6: AT+C6

Channel 7: AT+C7

Channel 8: AT+C8

Toggle (Self-locking)

Channel 1: AT+T1

Channel 2: AT+T2

Channel 3: AT+T3

Channel 4: AT+T4

Channel 5: AT+T5

Channel 6: AT+T6

Channel 7: AT+T7

Channel 8: AT+T8

Latch (Inter-locking)

Channel 1: AT+L1

Channel 2: AT+L2

Channel 3: AT+L3

Channel 4: AT+L4

Channel 5: AT+L5

Channel 6: AT+L6

Channel 7: AT+L7

Channel 8: AT+L8

Momentary (Non-locking)

Channel 1: AT+M1

Channel 2: AT+M2

Channel 3: AT+M3

Channel 4: AT+M4

Channel 5: AT+M5

Channel 6: AT+M6

Channel 7: AT+M7

Channel 8: AT+M8

Delay

Channel 1: AT+D1=XXXX

Channel 2: AT+D2=XXXX

Channel 3: AT+D3=XXXX

Channel 4: AT+D4=XXXX

Channel 5: AT+D5=XXXX

Channel 6: AT+D6=XXXX

Channel 7: AT+D7=XXXX

Channel 8: AT+D8=XXXX

XXXX refers to the 0000 to 9999 figures, Unit is seconds

All Relays Open

AT+AO

All Relays Close

AT+AC

Return command : OpenX, CloseX (X = 1/2/3/4/5/6/7/8/A)

Example 1:

Send command "AT+D1=0010", Channel 1 is "Open", after delay of 10 seconds, channel 1 is "Close"

Send command "AT+D2=0100", Channel 2 is "Open", after delay of 100 seconds, channel 2 is "Close"

Example 2:

Send command "AT+L1", Channel 1 is "Open", other Channels is "Close"

Send command "AT+L2", Channel 2 is "Open", other Channels is "Close"

Read input channel status:

Input channel 1: AT+I1

Input channel 2: AT+I2

Input channel 3: AT+I3

Input channel 4: AT+I4

Input channel 5: AT+I5

Input channel 6: AT+I6

Input channel 7: AT+I7

Input channel 8: AT+I8

Return: ONx, OFFx ((x=123…8)

Example 1 Read input channel 1, channel 1 has input Send command: AT+I1, return ON1

Example 2 Read input channel 2, channel 2 has no input Send command: AT+I2, return OFF2

Set input and output relationship (8 channels set together):

AT+G0 Unrelated ;                   return: OK0

AT+G1 Self-locking relationship (default); return: OK1

AT+G2 Interlocking relationship;        return: OK2

AT+G3 Momentary relationship;        return: OK3

Other values are invalid, no return

8-byte command (HEX format)

Note : The data must be in HEX format.

9600 Band ,8 Data bits,None Parity,1 Stop Bit。This commands has no delay function

Control command format:

| Bytes Number | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frame head |  | Reserved bytes |  |  | Channel | Command |  | checksum |  |
| Reading status | 0x5556 |  | 0x000000 |  |  | 0x01 Channel 1 0x02 Channel 2 0x03 Channel 3 0x04 Channel 4 0x05 Channel 5 0x06 Channel 6 0x07 Channel 7 0x08 Channel 8 | 00 |  | 0xAB 0xAC 0xAD 0xAE 0xAF 0xB0 0xB1 0xB2 |  |
| Relay open | 0x5556 |  | 0x000000 |  |  | 0x01 Channel 1 0x02 Channel 2 0x03 Channel 3 0x04 Channel 4 0x05 Channel 5 0x06 Channel 6 0x07 Channel 7 0x08 Channel 8 | 01 |  | 0xAD 0xAE 0xAF 0xB0 0xB1 0xB2 0xB3 0xB4 |  |
| Relay close | 0x5556 |  | 0x000000 |  |  | 0x01 Channel 1 0x02 Channel 2 0x03 Channel 3 0x04 Channel 4 0x05 Channel 5 0x06 Channel 6 0x07 Channel 7 0x08 Channel 8 | 02 |  | 0xAE 0xAF 0xB0 0xB1 0xB2 0xB3 0xB4 0xB5 |  |
| Relay toggle | 0x5556 |  | 0x000000 |  |  | 0x01 Channel 1 0x02 Channel 2 0x03 Channel 3 0x04 Channel 4 0x05 Channel 5 0x06 Channel 6 0x07 Channel 7 0x08 Channel 8 | 03 |  | 0xAF 0xB0 0xB1 0xB2 0xB3 0xB4 0xB5 0xB6 |  |
| Relay Interlock | 0x5556 |  | 0x000000 |  |  | 0x01 Channel 1 0x02 Channel 2 0x03 Channel 3 0x04 Channel 4 0x05 Channel 5 0x06 Channel 6 0x07 Channel 7 0x08 Channel 8 | 04 |  | 0xB0 0xB1 0xB2 0xB3 0xB4 0xB5 0xB6 0xB7 |  |
| Relay momentary | 0x5556 |  | 0x000000 |  |  | 0x01 Channel 1 0x02 Channel 2 0x03 Channel 3 0x04 Channel 4 0x05 Channel 5 0x06 Channel 6 0x07 Channel 7 0x08 Channel 8 | 05 |  | 0xB1 0xB2 0xB3 0xB4 0xB5 0xB6 0xB7 0xB8 |  |
| All Relays Open | 0x5556 |  | 0x000000 |  |  | 0X00 All Relay | 07 |  | B2 |  |
| All Relays Close | 0x5556 |  | 0x000000 |  |  | 0X00 All Relay | 08 |  | B3 |  |
| Read input port status | 0x5556 |  | 0x000000 |  |  | 0x01 IN1 0x02 IN 2 0x03 IN 3 0x04 IN 4 0x05 IN 5 0x06 IN 6 0x07 IN 7 0x08 IN 8 | 09 |  | 0xB5 0xB6 0xB7 0xB8 0xB9 0xBA 0xBB 0xBC |  |
| Input and output relationship | 0x5556 |  | 0x000000 |  |  | 0x0000 Unrelated 0x0001 Self-locking relationship (default) 0x0002 Interlocking relationship 0x0003 Momentary relationship | 0A |  | 0xB5 0xB6 0xB7 0xB8 0xB9 0XB9 |  |

5 Control commands：

1. Reading status（reading the satus of the relay (on/off)）

Channel 1 ：55 56 00 00 00 01 00 AC

Channel 2 ：55 56 00 00 00 02 00 AD

Channel 3 ：55 56 00 00 00 03 00 AE

Channel 4 ：55 56 00 00 00 04 00 AF

Channel 5 ：55 56 00 00 00 05 00 B0

Channel 6 ：55 56 00 00 00 06 00 B1

Channel 7 ：55 56 00 00 00 07 00 B2

Channel 8 ：55 56 00 00 00 08 00 B3

2. Relay open (issue this command ，Relay open , COM connect to NO ）

Channel 1 ：55 56 00 00 00 01 01 AD

Channel 2 ：55 56 00 00 00 02 01 AE

Channel 3 ：55 56 00 00 00 03 01 AF

Channel 4 ：55 56 00 00 00 04 01 B0

Channel 5 ：55 56 00 00 00 05 01 B1

Channel 6 ：55 56 00 00 00 06 01 B2

Channel 7 ：55 56 00 00 00 07 01 B3

Channel 8 ：55 56 00 00 00 08 01 B4

3. Relay close (issue this command ，Relay close , COM disconnect NO , and COM connect to NC ）

Channel 1 ：55 56 00 00 00 01 02 AE

Channel 2 ：55 56 00 00 00 02 02 AF

Channel 3 ：55 56 00 00 00 03 02 B0

Channel 4 ：55 56 00 00 00 04 02 B1

Channel 5 ：55 56 00 00 00 05 02 B2

Channel 6 ：55 56 00 00 00 06 02 B3

Channel 7 ：55 56 00 00 00 07 02 B4

Channel 8 ：55 56 00 00 00 08 02 B5

4. Relay toggle（Relay status reversal，if COM connect to NO，this commands will Disconnect COM to NO and Reverse COM connect to NC，and vice versa）

Channel 1 ：55 56 00 00 00 01 03 AF

Channel 2 ：55 56 00 00 00 02 03 B0

Channel 3 ：55 56 00 00 00 03 03 B1

Channel 4 ：55 56 00 00 00 04 03 B2

Channel 5 ：55 56 00 00 00 05 03 B3

Channel 6 ：55 56 00 00 00 06 03 B4

Channel 7 ：55 56 00 00 00 07 03 B5

Channel 8 ：55 56 00 00 00 08 03 B6

5. Relay Interlock

Channel 1 ：55 56 00 00 00 01 04 B0

Channel 2 ：55 56 00 00 00 02 04 B1

Channel 3 ：55 56 00 00 00 03 04 B2

Channel 4 ：55 56 00 00 00 04 04 B3

Channel 5 ：55 56 00 00 00 05 04 B4

Channel 6 ：55 56 00 00 00 06 04 B5

Channel 7 ：55 56 00 00 00 07 04 B6

Channel 8 ：55 56 00 00 00 08 04 B7

6. Relay momentary（Relay COM connect to NO，disconnect after 500MS ）

Channel 1 ：55 56 00 00 00 01 05 B1

Channel 2 ：55 56 00 00 00 02 05 B2

Channel 3 ：55 56 00 00 00 03 05 B3

Channel 4 ：55 56 00 00 00 04 05 B4

Channel 5 ：55 56 00 00 00 05 05 B5

Channel 6 ：55 56 00 00 00 06 05 B6

Channel 7 ：55 56 00 00 00 07 05 B7

Channel 8 ：55 56 00 00 00 08 05 B8

7 All Relays Open

55 56 00 00 00 00 07 B2

8 All Relays Close

55 56 00 00 00 00 08 B3

Once issue a command, will have a return fame , 7th byte of return fame mean the satus of realy .

Return command format:

| Bytes Number | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frame head |  | Reserved bytes |  |  | Channel | Command | checksum |
| Return relay open | 0x333C |  | 0x000000 |  |  | 0x01 Channel 1 0x02 Channel 2 0x03 Channel 3 0x04 Channel 4 0x05 Channel 5 0x06 Channel 6 0x07 Channel 7 0x08 Channel 8 | 01 | 0x71 0x72 0x73 0x74 0x75 0x76 0x77 0x78 |
| Return all Relay open | 0x333C |  | 0x000000 |  |  | 0X00 All Relay | 01 | 0X70 |
| Return relay close | 0x333C |  | 0x000000 |  |  | 0x01 Channel 1 0x02 Channel 2 0x03 Channel 3 0x04 Channel 4 0x05 Channel 5 0x06 Channel 6 0x07 Channel 7 0x08 Channel 8 | 02 | 0x72 0x73 0x74 0x75 0x76 0x77 0x78 0x79 |
| Return all Relay close | 0x333C |  | 0x000000 |  |  | 0X00 All Relay | 02 | 0X71 |

2 return command

1、Return relay open（return this command，mean COM connect to NO ）

Channel 1 ：33 3C 00 00 00 01 01 71

Channel 2 ：33 3C 00 00 00 02 01 72

Channel 3 ：33 3C 00 00 00 03 01 73

Channel 4 ：33 3C 00 00 00 04 01 74

Channel 5 ：33 3C 00 00 00 05 01 75

Channel 6 ：33 3C 00 00 00 06 01 76

Channel 7 ：33 3C 00 00 00 07 01 77

Channel 8 ：33 3C 00 00 00 08 01 78

2、Return relay close（return this command，mean COM disconnect NO , and COM connect to NC ）

Channel 1 ：33 3C 00 00 00 01 02 72

Channel 2 ：33 3C 00 00 00 02 02 73

Channel 3 ：33 3C 00 00 00 03 02 74

Channel 4 ：33 3C 00 00 00 04 02 75

Channel 5 ：33 3C 00 00 00 05 02 76

Channel 6 ：33 3C 00 00 00 06 02 77

Channel 7 ：33 3C 00 00 00 07 02 78

Channel 8 ：33 3C 00 00 00 08 02 79

Read input channel status:

Read IN1 : 55 56 00 00 00 01 09 B5

Read IN2 : 55 56 00 00 00 02 09 B6

Read IN3 : 55 56 00 00 00 03 09 B7

Read IN4 : 55 56 00 00 00 04 09 B8

Read IN5 : 55 56 00 00 00 05 09 B9

Read IN6 : 55 56 00 00 00 06 09 BA

Read IN7 : 55 56 00 00 00 07 09 BB

Read IN8 : 55 56 00 00 00 08 09 BC

1. Return: (NPN Input, Input On)

Return IN1 On : 33 3C 00 00 00 01 81 F1

Return IN2 On : 33 3C 00 00 00 02 81 F2

Return IN3 On : 33 3C 00 00 00 03 81 F3

Return IN4 On : 33 3C 00 00 00 04 81 F4

Return IN5 On : 33 3C 00 00 00 05 81 F5

Return IN6 On : 33 3C 00 00 00 06 81 F6

Return IN7 On : 33 3C 00 00 00 07 81 F7

Return IN8 On : 33 3C 00 00 00 08 81 F8

2. Return: (NPN Input, Input Off)

Return IN1 Off: 33 3C 00 00 00 01 82 F2

Return IN2 Off: 33 3C 00 00 00 02 82 F3

Return IN3 Off: 33 3C 00 00 00 03 82 F4

Return IN4 Off: 33 3C 00 00 00 04 82 F5

Return IN5 Off: 33 3C 00 00 00 05 82 F6

Return IN6 Off: 33 3C 00 00 00 06 82 F7

Return IN7 Off: 33 3C 00 00 00 07 82 F8

Return IN8 Off: 33 3C 00 00 00 08 82 F9

Set input and output relationship (8 channels set together):

Unrelated                    :55 56 00 00 00 00 0A B5

Self-locking relationship (default) : 55 56 00 00 00 01 0A B6

Interlocking relationship        : 55 56 00 00 00 02 0A B7

Momentary relationship         : 55 56 00 00 00 03 0A B8

Other values are invalid, no return